Hi,

As I see the code wasn’t as simple as I thought. So I will try to explain step by step. I hope the text will help you.

# Communication principle

So, you have arduino board and the pc connected virtually via RS232. If you want to send a message, the PC must build the message and send chars via RS232, the arduino have to receive the massage, process information inside and send back to the pc acknowledgement (ACK).

The codes you have are both side of this communication. On pc side it is code in visual studio (CArdCom.h/cpp, Serial.h/cpp + usage example in Comunication.cpp).  On arduino side is it code in Com.pde.

# First how it’s done

You have data you want to send (e.g. “HELLO”) and the type of the message (e.g. MSG).  In this implementation you must enclose this data into a communication frame. Communication frame has start symbol (its number 1 in hexadecimal code, in C written like 0x01), type+data, stop symbol (0x02), parity for error checking  (in fact we are not using it) and finaly end symbol (0x03).

(!! Hexadecimal numbers can be written like e.g. 0x4E, but in C meaning it is one BYTE, in this case is it representation of symbol “N” , see ASCII Table, and can be in range 0-255 (0x00-0xFF))

So the message you are sending looks this way (hexadecimal numbers are one symbol in fact):

0x01 MSGHELLO 0x02 0 0x03

The arduino receive the message and will process it, to let know the pc that the message was received it will send back ACK message:

0x01 ACK 0x02 0 0x03

Ok, its theory, now the real implementation and usage. I will try to explain on your problem:

{pin=22; digitalWrite(pin,HIGH);     pinMode(pin, OUTPUT);}

{pin=2; int brightness =255;    analogWrite(pin, brightness);}

# PC Side

In visual studio you need files CArdCom.h/cpp, Serial.h/cpp.

Serial.h/cpp – it handling RS232 connection

CArdCom.h/cpp – it handling message sending, ACK and other stuff

I expect there you know how to configure new project and include necessary files (in other case Google will help you  ).  All functions return false if they are unsuccessful (no ACK returning or no connection at all)

On the beginning you need to create communication class

CArdCom TEST;

Initialize communication - speed and port (port depends on you pc settings, arduino default use speed 115200 bauds)

TEST.Init("COM2",115200,8,NOPARITY,ONESTOPBIT);

Now you can send the message which telling to set hugh pin 22

TEST.SetPin(22);

Or set to low

TEST.ResetPin(22);

(this are short versions of functions, it automaticly uses message type SET or RES, so using TEST.SendMessageArd("SET22") or TEST.SendMessageArd("RES22") have the same efect)

Sending light brightness is litle tricky. Lets say the message type will be LIG and we have 3 lights. You nedd to build up messge and send it. Building up message means make string containing type+data:

char message[20]; //space for data, it can hold only 20-1 characters max!!

sprintf_s(message,20,"LIG%c%c%c",0x12,0xFF,0xA1); //building message

TEST.SendMessageArd(message); //sending message

The %c means that this place in the message will be replaced with one BYTE of value behind in the order as writen. Of course you can use decimal numbers it is equivalent: sprintf_s(message,20,"LIT%c%c%c",12,255,161);

There is place for warrning! Values 1,2,3 are used for communication frame (start, stop, end)  and 0 is used by C language to determine end of data in string, sou you cant use this values in light message and you are limited in range 4-255 (0x04-0xFF).

Now its all you need to know on pc, all messages are build up and send via RS232, if everything worked ACK was received and functions returned true. This example is in file Comunication.cpp. Lets go to arduino to understand what happening there.

# Arduino Side

Receiving messages is quiet complex, the code for reciiving is in file Com.pde , everything you need is to modify some minor parts. You can just copy all the code and use it.

### Litle theory

In arduino is implemented class Comunication, on arduino start is called function Init() which sets the speed to 115200. The function HandlMessage() is periodically called in main arduino loop.

If message on RS232 occurred, it is processed in function ReceiveMessage()  and after end symbol  is passed to function  ProcessMessage(), there is determined type of the message (type of the message is obtained by calling GetIdef() – the variable idef contains message type string after calling).  And actions will be done according to message type.

### In code

First you want to set pin 22 and you received corresponding message. This case is already implemented inside function  ProcessMessage():

if(strcmp("SET",idef) == 0)   		//if the message type is SET

{

Set(GetBody());			//Set the pin (GetBody() returns number of pin in message)

// digitalWrite(GetBody(), 1);  have the same effect

processed = 1;			//processing is done

Send("ACK");			//let the pc know we received and processed

}

Ok and the pin 22 is set, as we asked from PC by calling TEST.SetPin(22). The reset function is the same. I thing you no need to change on this two functions.

Light intensity is again little tricky. Firs the decision of the message, its again done in ProcessMessage():

if(strcmp("LIG",idef) == 0) 		//if the type is light message

{

DoLight();				//process it

processed = 1;

Send("ACK");			//let the pc know

}

Your main interest is function DoLight(). There you must get numbers out of the message and sets the pins. There is the little modification you have to do.

void Comunication:: DoLight()

{

int i = 0;				//counter

char LED[100];			//place for 100 values

//cycles until end symbol in message, +3 because first 4 symbols are start + type (LIG) , message is // filed by previous message processing, don’t worry about it

while((message[i+3] != END))

{

LED[i] = message[i+3];      		//coping brightness values to LED field

i++;					//icrementing

}

//code below you have to add and modify for your usage, e.g. we now have 3 values we send

//from PC stored in LED on 0,1,2

analogWrite(1, LED[0]);

analogWrite(2, LED[1]);

analogWrite(3, LED[2]);

//or faster

for(int j=0 ; j<i ; j++)  analogWrite(j+1, LED[j]);

}

And its all, of course you will use more values and analog pins and other adreses , but this is a principle. You need just modify the code in Com.pde.  Practically only function DoLight().

I hope it help.

Again warning! The codes are part of older work and were deeply modified, maybe there can be some error o errors inside. We don’t test it.

If you want some user GUI in window, it’s another story. Maybe latter, now focus to handle this.