Smart Climate Controller

ECE 445 Senior Design

Design Review

June 30, 2005

Project Team

Robin O

Teaching Assistant:

Timothy O’Connell

DESIGN SCHEMATICS

Figure 1: Overall Block Diagram

Figure 3: Software Flow Chart

MECHANICAL DRAWINGS

Dimensions: 12”x12”x12”

Volume: 1728 cubic inches

Figure 4: Model Type 1

Dimensions: 12”x12”x6”

Volume: 864 cubic inches

Figure 5: Model Type 2

Dimensions: 6”x6”x6”

Volume: 216 cubic inches

Figure 6: Model Type 3

TESTING PROCEDURES

Sensor Tests:

- CO2: as a function of Source Magnitude

- CO2: as a function of Environment Volume

- CO2: as a function of Response Time

- CO2: as a function of Time Duration

- Temperature: as a function of Error

Source Magnitude:  Initially, a relation between the magnitude of the CO2 source and the sensor’s reading will be determined.  Because of difficulties with obtaining consistent samples of known concentrations of CO2, this test will provide comparable information for further sensor testing discussed below.  As a consistent source, CO2 will be exhaled through a straw into an enclosed volume for differing durations.  Using Model Type 2 shown above in Figure 5, CO2 will be exhaled in 2 second, 4 second, and 6 second durations.  Five trials will be taken for each duration while the enclosure is vented thoroughly between trials.  In each trial, the CO2 sensors output will be recorded two minutes after the CO2 has been released into the volume to allow for proper acquisition.  Approximate theoretical values can be calculated and used to examine the accuracy of the sensor.  The resulting data will be constructed into graphs relating the sensor output voltage as a function of different durations of CO2 exhalation in a constant volume.

Environment Volume:  With the above data in hand, the reaction of the CO2 sensor against environment volume will be tested.  Having already conducted tests with the Model Type 2 volume, only the remaining volumes, Model Types 1 and 3 will be tested.  Using the most efficient and comparable exhalation duration determined in the prior tests against source magnitude, five trials per model will be tested with proper venting before each trial.  The data acquired will be constructed into graphs relating the CO2 sensor output against the volume of the enclosure with constant exhalation durations.  These data can be accessed further to predict performance in the scale of a vehicle interior.

Response Time:  Here, the CO2 sensor will be tested for response time.  Using only the Model Type 2 volume and the most efficient exhalation duration found, the sensor output readings will be taken at 20 seconds, 40 seconds, 60 seconds, and 80 seconds after the initial exhalation.  Five such trials will be taken with proper ventilation between trials.  The results found here will be compared to the output readings found in the first test conducted.  More specifically, these results will be compared to the readings taken two minutes after the initial exhalation during the tests against source magnitude.  The data acquired will be used to create graphs relating the sensor accuracy against the response time allotted in a constant volume.  The most efficient response time found here will then be implemented throughout the software.

Time Duration:  The CO2 sensor will be tested for consistency over time and to observe any drift in performance.  For ten minutes, two second exhalations will be introduced into the Model Type 2 volume at the turn of each minute.  The sensor output will also be recorded each minute.  The data acquired will be used to create graphs relating the sensor output voltage against time in a constant volume.  Any drift from theoretical output voltage determined from previous tests will be noted.

Error:  The temperature sensor will be tested for accuracy during these tests.  The readings from the temperature sensor will simply be compared to that of a thermometer situated in the same environment or enclosure.  Ten different sets of data will be recorded for comparison.  The results will be used to create graphs relating the sensor output, the actual temperature, and the error calculated.

TOLERANCE ANALYSIS

As suggested earlier in the section, the most important component of the product is the Carbon Dioxide Sensor.  It is the biggest factor regarding proper functionality of the SCC system.  The sensor must be sensitive enough to detect the slightest increase in levels of CO2 concentration in both a smaller emulated environment and also in an automobile interior.  This is especially the case with babies where the release of CO2 is relatively low.  Rough ratios of environment size to the size of the CO2 releasing agent will be taken into consideration and tested thoroughly.  It is believed that following the tests mentioned in the Testing Procedures section will result in a much better notion of how to achieve the optimum performance.

More specifically, to measure the sensitivity of the CO2 sensor to the increase in levels of CO2 concentration, the following test will be run.  Five tests will be run using the Model Type 2 volume shown in Figure 5.  For each test, the CO2 sensor will be run continuously for ten minutes while observing its output voltages.  Sensor readings will be taken and recorded every minute for that period of time.  After the first three minutes pass, air, and thus CO2, will be exhaled into the enclosed area for three seconds using a straw.  Data obtained will show the sensor output each minute during the ten minutes of each test.  It will also show when a new sample of CO2 was introduced, and if and when this increase was detected by the sensor.  Five such trials will be conducted with sufficient venting between each trial.  Graphs will be constructed relating sensor reading against time in a consistent volume.  If increases in levels of CO2 concentration are detected, the time needed to detect the increase will also be noted.  If no such increase was detected by the sensor throughout the ten minutes, the exhalation duration will be increased by two seconds.  This will repeat if results consistently show no increase in CO2 levels.  Using such data, it is possible to detect to what level of sensitivity the CO2 sensor can sense increases in concentration levels.  Because this project focuses on detecting increases in CO2 levels and not on the actual concentration values, these tests are thought to be sufficient ratings for the sensor.

PARTS COST ANALYSIS & STATUS

| Description | Part Number | Manufacturer | Qty. | Status | Cost |
| --- | --- | --- | --- | --- | --- |
| CO2 Sensor | TGS4161 | Figaro Engineering | 1 | On Hand | $35.25 |
| Temperature Sensor | LM19CIZ | National Instruments | 1 | On Hand | $0.18 |
| Operational Amplifier | TLC271 | Texas Instruments | 1 | On Hand | $0.27 |
| Microcontroller | PIC16F877A | Microchip | 1 | On Hand | $4.68 |
| Oscillator 20.0MHz | F1100E-0140 | Fox Electronics | 1 | On Hand | $1.58 |
| Resistors: 10 Ohm 100 Ohm 120 Ohm 150 Ohm 470 Ohm 10000 Ohm | 25J10R 25J100R 25J120R 25J150R 4308R2471 25J10K | Ohmite Ohmite Ohmite Ohmite Bourns Ohmite | 1 1 4 1 1 1 | On Hand | $6.49 |
| Yellow LED 5mm 30 Degrees | HLMP3401 | Hewlett-Packard | 4 | On Hand | $0.60 |
| SPST 5-POS DIP Switch | 76PSB05 | Grayhill | 1 | On Hand | $1.26 |
| 12”x12” Foamboards | 858277 | Royal Consumer Products | 10 | To Be Ordered | $10.44 |
| 12”x12” Vinyl Sheets | NA | Reynolds Consumer Products | 3 | On Hand | $2.49 |
| 5.6 V Zener Diode | D1N4734A | General Semiconductor | 3 | On Hand | $0.30 |
| 0.1uF Capacitor | CK104 | BC Components | 5 | On Hand | $1.27 |

SCHEDULE

| Week | Description of Activities |
| --- | --- |
| 6/20 | Order CO2 sensor – Modules from Telaire or Figaro - Get temperature sensor from ECE Parts Shop – Western Electic - Begin assembly of simulation box – From Illini Plastics or Machine Shop 5 sheets of 1/8” 12” x 12” clear acrylic Design Sensor Circuits – Meeting specifications of datasheets Begin coding of PIC16F877A – Recognizing sensor inputs via ADC & outputting to Operations block |
| 6/27 | - Test CO2 and temperature sensors thoroughly following Testing Procedures – Find optimum testing parameters: time, distance from source, environment size - Finish coding PIC16F877A – Test for proper functionality - Begin circuit build – Modules: Execution, Sensor Circuits, Microcontroller, and Operations Research CO2 – Concentration of in exhalation of child, determine sensitivity of CO2 sensor Obtain crucial parts – LM19CIZ, TLC271, TGS4161, F1100E, & PIC16F877A |
| 7/4 | - Complete build of Models – 1, 2, and 3 using foamboard & vinyl - Complete actual build – Attach sensors to box and finish circuitry - Debugging – Test for accuracy and proper sequencing of events/operations |
| 7/11 | - Debugging and final tests – Test for Performance Requirements |