# SMART CLIMATE CONTROLLER

By

Robin O

ECE 445, SENIOR DESIGN PROJECT

SUMMER 2005

TA: Timothy O’Connell

July17, 2005

Project 6

# ABSTRACT

I have designed and created a Smart Climate Controller (SCC), to automatically monitor and protect passengers left unattended in vehicles.  The SCC uses a combination of a carbon dioxide sensor and a temperature sensor to gather information on conditions within the vehicle and to provide the proper ventilation and other actions.  Due to complications with implementing the system into an actual vehicle, a smaller model has been created, tested, and analyzed in comparison to what an actual model would be.

This system uses an electrolyte based carbon dioxide sensing mechanism to detect the presence of passengers within the vehicle.  After various tests and characterizations, it was found to be ideal for use with the SCC.

The design worked but there is some room for improvements and or expansions.  The cost of the final implementation was much lower than the initial expectations.  Discussions on such matters are presented in the Conclusion.

# TABLE OF CONTENTS

1.	INTRODUCTION	1

1.1 Purpose	1

1.2 Specifications	2

1.3 Subprojects	2

1.3.1 Execution Module	2

1.3.2 Sensor Circuits Module	2

1.3.3 Microcontroller Module	2

1.3.4 Operations Module	2

2.	DESIGN PROCEDURE	3

2.1 Carbon Dioxide Sensor Circuit Design	3

2.2 Temperature Sensor Circuit Design	4

2.3 Microcontroller Circuit Design	4

2.4 Operations Design	5

3.	DESIGN DETAILS	6

3.1 Execution Module	6

3.2 Carbon Dioxide Sensor Circuit	6

3.3 Temperature Sensor Circuit	7

3.4 Microcontroller Module	8

3.5 Operations Module	8

4.	DESIGN VERIFICATION	10

4.1 Testing	10

4.1.1 Carbon Dioxide Sensor Against Source Magnitude	10

4.1.2 Carbon Dioxide Sensor Against Environment Volume	10

4.1.3 Carbon Dioxide Sensor Against Time Duration	10

4.2 Scaling And Approximating	11

4.3 Conclusions	12

5. 	COST	13

5.1 Parts And Labor	13

6. 	CONCLUSIONS	14

REFERENCES	15

APPENDIX A – BLOCK DIAGRAMS/FLOW CHARTS	Appendix.1

APPENDIX B – SCHEMATICS/MECHANICAL DRAWINGS	Appendix.3

APPENDIX C – MANUFACTURER SPECIFICATIONS/TEST DATA	Appendix.9

APPENDIX D – PICTURES	Appendix.23

# 1. INTRODUCTION

This report describes an automated climate controlling system, or a Smart Climate Controller (SCC).  I have designed and created the SCC such that it will protect unattended passengers left inside vehicles from extreme temperatures while the vehicle owner returns.  These passengers will be protected from the extreme temperatures within via controls to the vehicle ventilation system.  More specifically, once a driver key has been removed from the ignition, the SCC determines that the engine has been turned off and thus begins to monitor a carbon dioxide sensor placed within the interior of the vehicle.  When an increase is detected, the SCC determines a passenger is present and begins to monitor the temperature within the vehicle.  Once the temperature reaches a set upper limit, the SCC reacts by turning on the ignition and the air-conditioning.  The power doors and power windows are also locked and closed, respectively to deter theft while the engine is running.  While the temperature sensor is continuously monitored, if the temperature falls below a set lower limit, the air-conditioning alone is turned off.  The temperature is continuously monitored in this manner while the air-conditioning turns on and off at the command of the SCC until the driver key is reinserted into the ignition switch.  At this point, the SCC is reset and turned off and ready to be triggered again.  Due to difficulties and complexities in implementing the SCC into an actual vehicle, I have designed and created a smaller scaled model for testing.

  - Purpose

The purpose of this project was to utilize skills learned to engineer a product that would serve as a platform for an automated smart climate controller, or the SCC, that turns on ventilation systems in the presence of persons and or animals and turns these systems off in the absence of persons and or animals.  Such a system may be implemented in a variety of different settings.  Two of such settings comprise of a vehicle and a multi-room building.

In the case of a vehicle, the SCC would provide the proper ventilation needed to unattended passengers within the vehicle.  A carbon dioxide sensor used to detect the presence or lack of passengers would determine when or when not to begin the climate controlling process.

In the case of an in home or building application, the SCC would determine which rooms of the house are occupied by persons or animals.  Accordingly, proper climate controls will be offered only to occupied rooms while vacant room climate controls are temporarily shut down to greatly improve energy and cost efficiency.  Constant monitoring of carbon dioxide levels in each room will quickly determine when the occupants have moved out of or into another room and thus adjust the climate controls accordingly.

For purposes of the demonstration, I have selected to direct the automated climate controller towards the vehicle implementation.  In previous and related projects, I have seen numerous alerting systems to signal that an unattended passenger is inside a vehicle.  However, I felt that alarms merely annoy individuals and after a quick glance, people usually look away.  In the meantime, nothing is being done to help the passenger within.  While temperatures inside vehicle can reach 120°F in just 20 minutes [1] on a summer day, the passenger may possibly die before help arrives.  I also noticed that in previous projects, the method of detecting the presence of a passenger was via weight sensors or the like.  However, this can be unreliable and falsely perform actions when heavy objects are placed on the sensors.  Thus, with the SCC, I planned to alleviate these problems and make a more reliable and successful product.

  - Specifications

This project was split up into four main components or modules for simplicity as seen in Diagram A.1.  For individual module specifications, refer to Section 1.3.

In order for the SCC to function properly, the main concern is for the carbon dioxide sensor to correctly detect a sufficient increase in the level of carbon dioxide concentration inside the test environment.  The carbon dioxide sensor must also be able to detect such increases within a reasonable timeframe.  Timing is crucial because the longer it takes for this detection, the more risk there is to the health of the passenger.  In a real-life scenario, the temperature inside of a vehicle is known to increase dramatically within 20 minutes [1].  Using such a scale as reference, the ideal carbon dioxide sensor would need to make such detections within 5-8 minutes after engine shut off.

  - Subprojects

For simplicity, the project was divided into four basic modules which each perform specific tasks.  These modules are described more in detail below.

    - Execution Module

The Execution Module needs to emulate the insertion or the removal of the driver key from the ignition.  In other words, it needs to instruct the microcontroller to begin or stop the SCC.

    - Sensor Circuits Module

The Sensor Circuits Module needs to output to the microcontroller, both the carbon dioxide and temperature sensor readings for further processing.  The circuitry of this module must allow for a reasonable output range for proper analog to digital conversion once the information reaches the microcontroller.

    - Microcontroller Module

The Microcontroller Module is responsible for awaiting instructions from the Execution Module to begin the embedded software sequence for the SCC.  The flowchart of the implemented software is shown in Diagram A.2.  Once instructed to begin the sequence, it is responsible for monitoring the carbon dioxide sensor outputs for any significant increase in levels.  When such increases are detected, it needs to begin continual monitoring of the temperature sensor output.  Preferably, an inside temperature of about 80°F, should indicate high temperatures while 70°F indicates low temperatures.  While comparing actual temperature sensor outputs to these preset upper and lower limits, the microcontroller must turn on or off the air-conditioning.  It is also responsible for initially starting the ignition if not already on, while also locking the power door locks and closing the power windows.  Lastly, it needs to be able to sense when the Execution Module implies that the driver key has been reinserted into the ignition and to stop further processing accordingly.

    - Operations Module

The Operations Module is responsible for emulating the actions taken by the microcontroller and thus the embedded software of the SCC.  It needs to show using light-emitting diodes (LEDs), the state of the ignition and the air-conditioning, as well as the states of the power door locks and the power windows.

2.  DESIGN PROCEDURE

2.1 Carbon Dioxide Sensor Circuit Design

In determining the best way to detect the presence of a passenger inside of a vehicle, many solutions were examined.  The first option was to use a weight sensor built into the seats.  However, I realized that if heavy objects were placed on a seat, as is frequently the case in real life, the sensors would falsely alert that a passenger is present.  To rectify such a problem, I considered sensing movement with the weight sensor.  Frequent changes in the weight sensed would imply some movement on the seat.  This would imply the presence of a passenger.  However, this idea seemed to be more complex for such a simple application.  Tilt sensors were also considered for use in the seats.  Depending on the sensitivity of such sensors, I could detect any movement within the vehicle and conclude that a passenger is inside.  However, there was yet another barrier to the ideas of detecting movement.  If a passenger has fallen into a deep sleep and shows almost no movement, the SCC may falsely assert that a passenger is not present inside the vehicle.  Some latter options to resolve this problem was to monitor audio signals or to use a thermal sensor.  Both ideas were brought up by Professor Gary Swenson.  The former plan would monitor for audio of breathing while the latter plan picks up body heat, or detects blackbody radiation [2].  However, these ideas seemed to be too much complexity compared to the application.

After much consideration, a carbon dioxide sensor seemed to be the ideal method of detecting the presence of a passenger inside of a vehicle.  In any and all scenarios, a carbon dioxide sensor ensures proper detection regardless of problems dealing with motion and heavy objects.  Because all persons and animals exhale carbon dioxide into the air whether they are sleeping or awake, this sensor was deemed to be the sensor used for my project.

The ideal carbon dioxide sensor for my implementation needs to be able to detect an increase in carbon dioxide levels.  The exact value of the carbon dioxide in parts per million (ppm) is unnecessary for my application.  However, the sensor must be able to detect concentrations in a range with a lower limit of approximately 350 ppmCO2 or 0.03% CO2.  This is due to the fact that the average concentration of carbon dioxide in the atmosphere is at this level [3].  I deduced that in an enclosed area, the carbon dioxide concentration would not fall below this level without special treatment.  Also, this level would only increase with a breathing person present within.

The desired upper limit for the range of the carbon dioxide sensor concentrations sensed was initially unknown.  This was due to the many different factors and uncertainties at the stage of development.  First of which was the environment volume to be used for testing and also the average volume of a vehicle interior.  Although tests were not run inside of an actual vehicle, this information would be necessary to relate the scale of my project and tests to that of a vehicle interior later for comparisons and so forth.  Another such uncertainty was the amount of carbon dioxide I would use for testing and the amount of carbon dioxide a child would exhale into a vehicle for latter comparisons.  For testing, I was unsure of how to provide smaller, and yet, known values of carbon dioxide for later testing.  Some ideas suggested by Professor Jennifer Bernhard consisted of using small animals, invertebrates, and insects.  Although these options provide for a smaller and scaled testing environment, I still was unsure of the actual amount of carbon dioxide output.  After much research, it was very difficult to find such data for small animals and insects.  However, during the process, I was able to gather enough bits of information to make reasonable approximations to calculate the reference values I needed.  The details of this process and the specific findings are presented in the Design Verification and Conclusion.

In searching for a carbon dioxide sensor ideal for my purposes, I came upon many using a non-dispersive infrared (NDIR) absorption detection method.  To summarize, this method requires sampling via a tube-like passage wherein a light source emits light.  At the opposing end, an infrared detector observes attenuations from the original emissions from the light source [4].  Attenuations in infrared light are known to be proportional to the amount of carbon dioxide present and thus the carbon dioxide concentration is observed.  One concern this sensor brought to my attention was the price.  All such carbon dioxide sensors valued at a few hundred dollars per part.  The least expensive NDIR based carbon dioxide sensor I was able to find was the Telaire-6004 [5] selling for a bit over $100 per sensor.

Eventually I was able to find a considerably cheaper sensor, the Figaro-TGS4161, for $35 per sensor.  This sensor, unlike all others, used a different detection method using a solid electrolyte sensitive to carbon dioxide.  The details will be discussed in the Design Details section of this report.  To summarize, this sensor detects in the range of 350 – 5000 ppmCO2 [6].  From the approximations mentioned earlier, I discovered this range was ideal for all of my purposes of testing and for relating to a real-life scenario.  Although I was skeptical of its performance in terms of reaction time, reliability, and consistency, as I will discuss later in the Design Verification section, all these specifications were met.

As presented in Design Details, the carbon dioxide sensor itself outputs only minimal voltage ranges, ranges that are too low to efficiently implement to an analog to digital converter.  Thus I have attached a non-inverting amplifier circuit using an operation amplifier made by Texas Instruments, the TLC271.  There are many different operational amplifiers that I could have used for this application; however this one was used primarily because it was recommended by Figaro [6].

2.2 Temperature Sensor Circuit Design

There was not much time spent on choosing a temperature sensor for my project.  However, it seemed that there was not too much to consider in selecting a type of temperature sensor.  I first planned to use a thermistor.  However, with the assistance from my teaching assistant, Timothy O’Connell, I was convinced that a semi-conducting temperature sensor was more reliable and easier to implement.  The specific sensor used was the National Instruments LM19.

There were a few reasons for selecting the LM19.  The LM19 output voltage has a very linear relationship [7] with the temperature whereas some thermistors showed nonlinear relationships.  More detail of this will be presented in the Design Details section.  It is small, compact, and uses very little current.  The LM19 was also very inexpensive to purchase, however, free multiple samples were available.

2.3 Microcontroller Circuit Design

After considerations for the microcontroller were made, I decided upon using the Microchip PIC16F877A.  Most of my reasons for selecting this chip are related to more ease of use.  Some familiarity with the programming language and the chip directed me towards using this chip for my project.  It also provides a sufficient number of digital input and output ports along with analog to digital converters.  One of the key points about the SCC is that it should be expandable and adaptable for many different applications and environments.  The PIC16F877A offers plenty of extra available ports for such applications.  Although it is also quite inexpensive, I was again able to obtain free samples for use with the SCC.  There are also some important considerations with respect to the drive current provided by the microcontroller.  Such specifications will determine the drive capabilities of the PIC16F877A outputs.  Although the tested amounts for the purposes of this particular project were adequate, further assessment is needed to determine the probability of driving larger light sources and loads.  This will be discussed more in depth in the Design Details section of the report.

2.4 Operations Design

The design of the operations module consists of simple mapping all of the digital outputs of the microcontroller to LEDs for display.  Each of these LEDs represents a particular action or state of an element within the system.  These LEDs represent the states of the ignition, air-conditioning, power door locks, and power windows.

Each of these LEDs is driven by different digital microcontroller outputs and requires a certain range of allowable current.  Using the +5 V output provided by the microcontroller and resistors in series to the LEDs, these current ranges were maintained.  Details of these findings will be discussed in the Design Details section.

This simple structure is solely for the purposes of this SCC application, and may be as complex as desired for any other climate controlling applications.  For instance, for a real-life application, the LED representing the state of the air-conditioning may be replaced by an actual fan used for cooling.  However, as previously noted, there are significant considerations needed in using such larger loads.  The microcontroller drive current must be assessed for such applications.

3. DESIGN DETAILS

The following sections describe modular circuit details of the Smart Climate Controller with applications of necessary polarity and over-voltage protection already assumed.

3.1 Execution Module

The Execution Module was designed to emulate a driver key being inserted or removed from the ignition in a real-life scenario.  For the purposes of this particular project, a DIP switch was used placed in series with a resistance across +5 V and ground.  The circuitry is presented in the design schematic of Diagram B.1.  The value of the resistance was determined such that this switch would take in minimum current to run.  Using 15μA as the desired current draw from this module, the following resistance was found.

Rsw [Ω]= 5 [V] / (1.5 x 10-5 [A])		(3.1.1)

With this, Rsw was determined to be 330 kΩ.  With the configuration shown in the schematic of Diagram B.1, this switch when closed, feeds a logic high, or +5 V to the microcontroller input.  This tells the software that the driver key is still in the ignition so do nothing yet as shown in the flowchart of Diagram A.2.  When the switch is open, this feeds a logic low to the microcontroller input indicating that the key has been removed from the ignition and thus to start monitoring for passengers.  Although simple, the Execution Module is crucial to the proper functionality of the SCC.

3.2 Carbon Dioxide Sensor Circuit

As mentioned in the Design Procedure, the Figaro TGS4161 was used for this application of the SCC.  As shown in Diagrams B.2A and B.2B, this sensor has four pins.  Two of the pins are used heat a layer of a heater substrate attached to a layer of a solid electrolyte that is sensitive to carbon dioxide.  When power is applied to these two pins via a +5 V source, the heater layer presents a resistance of about 70 Ω and draws approximately 50 mA of current.  While the solid electrolyte is heated via the heater layer, it exhibits an electromotive force (EMF) and thus a voltage across its remaining two terminals [6].  This voltage is sensitive and changes with respect to the amount of carbon dioxide present in the atmosphere.  The steel gauze and metal cap shown in Diagram B.2A serves to molecularly filter out interference gases and thus forming an accumulation of carbon dioxide gas within the cap of the sensor [6].

Initially, without having tested the sensor for myself, I found from its documentation that the output voltage, or the EMF output, of the sensor at 350 ppmCO2 was rated anywhere from 220 mV to about 490 mV.  This is quite a large range of output voltage for one carbon dioxide concentration of 350 ppmCO2.  It also showed that this output voltage decreases as more carbon dioxide is introduced to the sensor.  More importantly, these output values were too small in size to be implemented into an analog to digital converter of the microcontroller.  The PIC16F877A has a potential of converting 0 V to 5 V voltages into digital entities.  Using such small values on these converters would result in poor resolutions it would be difficult to differentiate one reading from another.  Thus an amplifier was needed.

To determine the gain of the amplifier needed, the full range of the sensor output was needed.  As mentioned above, the EMF output at 350 ppmCO2 was anywhere between 220 mV to 490 mV but the documentations did not rate the expected minimum voltage output by the sensor.  Instead there was a rating in terms of ΔEMF rather than just EMF.  The rated ΔEMF at 3500 ppmCO2 varied between 44 mV to 72 mV.  According to Figaro [6], ΔEMF is expressed as in the equations below.

ΔEMF(X ppmCO2) [mV] = EMF(350 ppmCO2) [mV] – EMF(X ppmCO2) [mV]		(3.2.1)

ΔEMF(350 ppmCO2) [mV] = EMF(350 ppmCO2) [mV] – EMF(350 ppmCO2) [mV]		(3.2.2)

ΔEMF(350 ppmCO2) [mV] = 0 [mV]		(3.2.3)

The range of 44 mV to 72 mV is yet another quite large range for one specific rating at one specific concentration of carbon dioxide.  Thus taking the average of this range of values, I obtained an estimate ΔEMF of about 58 mV at 3500 ppmCO2.  Although the sensor has a full potential of reading up to 5000 ppmCO2, I used 3500 ppmCO2 for purposes of initially setting up an amplifier circuit.  All of these initially documented ratings for the Figaro TGS4161 are summarized in Table C.1 of Appendix C.

Knowing that the maximum EMF output probable was 490 mV, and that this value decreases with increasing concentration of carbon dioxide, I made plans to attach a non-inverting amplifier with a gain of about 10 V/V.  With such amplification, I can expect the total carbon dioxide sensor circuitry to output an EMF of about 4090 mV at most.  The initial amplifier circuit for this is shown in Diagram B.3.  To accommodate for a 10 V/V gain, as shown in the figure, the resistance Ri was set as 1000 Ω, while the Rf resistor was set as 10000 Ω.  A preliminary simulation for the amplifier was constructed as in Diagram C.2A and run while the output is as shown in Diagram C.2B.  As mentioned in the Section 2.1 of Design Procedures, the Texas Instruments TLC271 operational amplifier was used.

Gain [V/V] = Rf [Ω] / Ri [Ω]		(3.2.4)

Gain [V/V] = 10000 [Ω] / 1000 [Ω] = 10 [V/V]		(3.2.5)

However, as I will discuss later in the Design Verifications section of this report, during actual implementation of the amplification circuit to the carbon dioxide sensor, the output EMF was determined to be quite different from the ratings suggested by Figaro.  This may be due to a number of different factors.  The environment in which I initially tested the carbon dioxide sensor might not have been exactly at 350 ppmCO2 concentration.  This may be confirmed if I had used another carbon dioxide sensor set up in the same environment for comparison.  However, I did not have such financial access to do so at that point in time.  Also, the supply voltage applied, or the heater voltage, to the carbon dioxide sensor is known to have an effect on the output voltage.  Generally, as the supply voltage increases, the output voltage also increases in value.  The recommended supply voltage was 5.0 V give or take 0.2 V.  Although I had set the Agilent E3631A power supply at 5.0 VDC, there are some variations due to the additional circuitry involved while testing and varying amounts of currents drawn by the different components.  Nonetheless, for my applications, I only need to be able to detect the changes in carbon dioxide levels and not the actual value.  I then modified the amplifier circuit to have a gain of 24 V/V.  For this, using Equations 3.2.4, I was able to estimate resistance values of 1000 Ω for Ri and 24000 Ω for Rf.  The schematic of Diagram B.1 shows the circuit in its entirety including the modified amplifier.  The actual tested output EMF voltages are shown in Table C.3 with and without the modified amplifier circuit attached.  Specifics regarding how analog to digital conversions and resolutions were estimated have been described in Section 3.4.

3.3 Temperature Sensor Circuit

Using documentation provided by National Instruments, the LM19 was characterized in terms of output voltage and the temperature tested [7].  The data is shown in Table C.4A in table form with respect to temperatures in both degrees Celsius (°C), and degrees Fahrenheit (°F).  I formed a plot using this data and it is shown in Diagram C.4B.  As the figure and the linear fit suggests, there is an inverse relationship that is quite linear.  The relationship is more specifically shown as Equation 3.3.1 below.

Vout [mV] = (-6.544 [mV/°F]) ( Tf [°F]) + 2063 [mV]		(3.3.1)

With this I was able to facilitate future conversions between the LM19 output voltage to temperatures in degrees Fahrenheit and vice versa.  For my applications, I wanted the microcontroller to monitor for temperature levels of 79.5°F and 76.5°F.  Using Equation 3.2.5, these temperatures are represented by output voltages of about 1543.0 mV and 1562.5 mV, respectively.  Specifics regarding how analog to digital conversions and resolutions were estimated have been described in Section 3.4 below.

3.4 Microcontroller Module

In using the PIC16F877A, some configurations must be made within the software in terms of the ports and their characteristics.  For basic assignments regarding digital ports, the input and output ports are RB[1] and RC[0-3], respectively.

To set up the PIC16F877A to use as an analog to digital converter, four ports must be assigned in a specific manner.  As shown in Diagram B.1, ports RA[0] and RA[1] are used as inputs from the carbon dioxide sensor circuit and the temperature sensor, respectively.  Ports RA[2] and RA[3] are used as the analog to digital reference voltages [8].  Depending on these references, the resolution will change.  However, as I will discuss below, it was sufficient to use +5 V and ground as the VREF+ of RA[3] and VREF- of RA[2], respectively.

One of the main functions of the microcontroller is to convert analog signals passed on from the carbon dioxide and the temperature sensors to digital or binary numbers.  Before applying the sensor outputs to the analog to digital converter of the PIC16F877A, I calculated the expected resolution.

Resolution [mV/bit] = 5000 [mV] / 256 [bits]		(3.4.1)

Resolution [mV/bit] = 19.53 [mV/bit]		(3.4.2)

From Table C.3, the observed carbon dioxide sensor output ranges with the modified amplifier circuit attached were 4090 mV to about 2703 mV.  At this point, I decided that a resolution of about 20 mV/bit for such a range was sufficient.  The two temperatures of 79.5°F and 76.5°F which correspond to 1543.0 mV and 1562.5 mV, respectively, also are detectable with this resolution via an analog to digital converter.

3.5 Operations Module

As mentioned during the Design Procedure, this module is responsible for indicating what actions are executing and when they are executing.  For this application of the SCC, there are four main digital outputs coming from the microcontroller to the Operations Module.  Four different LEDs will light using these outputs, ports RC[0-3] on the PIC16F877A.  These LEDs were all yellow with diameters of 5 mm.  According to their documentations, the current draw ranges from about 20 mA to 30 mA [9].  In order to limit current drawn by the LEDs to this range, series resistors should be placed.  The value of these series resistors were calculated using Equation 3.5.1 below.

Rseries [Ω] = (Vsupply [V] – Vf [V]) / If [A]		(3.5.1)

Here, Rseries is the value of the resistor in series while Vsupply is the supply voltage.  Here, Vf and If denote the forward voltage and the desired current of the LED, respectively.  In the case of the yellow LEDs used here:

Vsupply [V] = 5 [V]		(3.5.2)

Vf [V] = 2.25 [V]		(3.5.3)

If [A] = 20 [mA]		(3.5.4)

The forward voltage of the yellow 5 mm LED ranges from 2.25 V to 2.60 V.  Here I have used 2.25 V. Using these values, the Rseries was determined to be 137.5 Ω.  But to be cautious, a 120 Ω resistor was used.

Rseries [Ω] = (5 [V] – 2.25 [V]) / 0.020 [A]		(3.5.5)

Rseries [Ω] = 137.5 [Ω] ≈ 120 [Ω]		(3.5.6)

From this, it appears using a resistor of this size results in a current of 23 mA drawn from the LED which is adequate for this application.

Although these specifications are fine for this SCC model project, there are some complications dealing with a real-life implementation.  These complications arise from the fact that many microcontrollers cannot drive more than 20 mA, or 25 mA in the case of the PIC16F877A.  However, simple solutions to these problems can be found using transistors.  For loads using less than 200 mA or so such as small lights or small motors, the 2N2222 transistor can be used [10].  Diagram B.3A shows a sketch of such an application to power a lamp.  For bigger loads needing more than 200 mA, a MOSFET such as the IRF540 may be used.  However, since this such standard MOSFETs require +10 V supplies to run at full capacity, a 2N2222 transistor or a 7406 converter may be used.  An example circuit is shown in Diagram B.3B.

4. DESIGN VERIFICATION

4.1 Testing

Overall, the main test that I wanted to pass was more qualitative than quantitative.  Once complete, the SCC for this application should be able to successfully run through the embedded software as a whole without any problems.  However, to be successful in this implementation, the most important factor comes from a correct characterization of the carbon dioxide sensor.  This is because there are many unknowns in dealing with this sensor.

4.1.1 Carbon Dioxide Sensor Against Source Magnitude

Initially, a relation between the magnitude of the carbon dioxide source and the sensor output was determined.  Because of difficulties with obtaining consistent samples of known concentrations of carbon dioxide, this test provides comparable information for further sensor testing outlined below.  As a consistent source, carbon dioxide was exhaled through a straw into an enclosed volume for differing durations.  Using Model Type 2 shown in Diagram B.4B, I exhaled in 4-second, 6-second, 8-second, and 10-second durations into the model.  Ten trials were taken for each duration period while the enclosure was vented thoroughly between trials.  In each trial, the minimum carbon dioxide sensor output, corresponding to the maximum concentration level, was recorded.  The results are outlined in table form in Table C.5A while plots of the data are shown as Diagrams C.6 through C.9.  From these results, averages were calculated and presented in the text boxes of each of plot.  Using approximations and from these results, averages were calculated and presented in the text boxes of each plot.

4.1.2 Carbon Dioxide Sensor Against Environment Volume

With the above data in hand, the reaction of the carbon dioxide sensor against environment volume was tested.  Having already conducted tests with the Model Type 2 volume, Model Type 1 shown in Diagram B.4A was tested.  As in the test described in Section 4.1.1, ten trials per exhalation duration periods, 4-second, 6-second, 8-second, and 10-second durations, were tested with proper venting between trials.  The data acquired was formed into a table as Table C.5B.  Data for the 4-second and 6-second durations show no data because there were no increases sensed by the carbon dioxide sensor in the increased volume.  Plots were also formed for these data and were overlaid onto Diagrams C.8 and C.9 for easier comparison to those of the Model Type 2 volume.  Once again, from these results, averages were calculated and presented in the text boxes of each plot.

The average output voltage values of each test duration and volume were plot onto Diagram C.10 for yet another comparison of performance between the two volumes.  From this plot, it was apparent that I could manage to get a significantly better reaction with a shorter exhalation from the Model Type 2 volume.  From this point onward, only Model Type 2 was used for further testing and characterizing of the carbon dioxide sensor.

4.1.3 Carbon Dioxide Sensor Against Time Duration

The carbon dioxide sensor was also tested for consistency over time.  Using only the Model Type 2 volume and a 6-second exhalation duration, five trials were run for 140 seconds to find the sensor behaviors with respect to time.  With sufficient venting between trials, time was started after the 6 seconds of exhalation with a stopwatch.  Every ten seconds thereafter until the full 140 seconds, I recorded the output voltage measured by a digital voltmeter.  The actual data obtained is shown as Table C11A while the plot of each trial overlaid is shown as Diagram C.11B.  The plot depicted in a bolder line indicates the average response of all five trials.  The vertical line shown at the 60 second mark indicates the minimum output voltage recorded.  This minimum voltage recorded also indicates the maximum carbon dioxide concentration sensed.  It shows how consistent the sensor is in terms of response time.

4.2 Scaling And Approximating

This section serves to relate and compare the smaller scaled-down project of the SCC described throughout this report to a real-life scenario.  Although success of this project is important, I feel it is also important to present what these results imply in terms of a real-life application.  To do this, a lot of research has been done.  Many times, however, the data I wanted to find were not available and thus approximation and or scales were needed.  This section walks through these approximations and shows my findings.

While researching, I found information regarding average breathing rates of different age groups.  This is shown in Table C.12A.  The important statistics here are that of infants and adults which are 20-30 breaths per minute and 14-18 breaths per minute, respectively [11].  This is because in the scaled-down project, I, an adult, am breathing into a box while in a real-life case, an infant is assumed to be breathing inside of a vehicle.  Since the SCC in this case is projected towards saving lives of young children, I estimated an average age of four years.  A child of age four describes an infant age group.

I also found that the average adult exhales about 42500 ppmCO2 per breath [12].  With this fact and the fact that an average adult breathes at a rate of 16 breaths per minute, I was able to make the following relationship in terms of carbon dioxide concentration exhaled per second for an adult.

CO2 Exhale Rate[ppmCO2/s] = 16[breaths/min] x 1/60[min/s] x 42500[ppmCO2/breath]		(4.2.1)

CO2 Exhale Rate [ppmCO2/s] = 11333 [ppmCO2/s]		(4.2.2)

An average adult is known to exhale about 500 mL of air per breath [13].  Using this along with the breathing rate for adults shown above, I can calculate the average volume of air exhaled per second.

Air Exhale Rate [L/s] = 16 [breaths/min] x 1/60 [min/s] x 0.5 [L/breath]		(4.2.3)

Air Exhale Rate [L/s] = 0.133 [L/s]		(4.2.4)

So in one second, the average adult exhales about 11333 ppmCO2 in about 0.133 L of air.

Vmodel2 [cubic inches] = 864 [cubic inches]		(4.2.5)

Vmodel2 [L] = 14.2 [L]		(4.2.6)

Using these conversions and the volume of Model Type 2, I was able to estimate the carbon dioxide concentration exhaled during the tests against source magnitude.  After converting the amount of carbon dioxide per 500 mL of air into the amount of carbon dioxide per volume of Model Type 2, I obtained a conversion factor of 106.7 ppmCO2/seconds.  Using this, I am able to estimate the following example.

CO2 After 6-Second Exhalation [ppmCO2] = 106.7 [ppmCO2/s] x 6 [s]		(4.2.7)

CO2 After 6-Second Exhalation [ppmCO2] = 640.2 [ppmCO2]		(4.2.8)

For relating my carbon dioxide exhalation rates to that of a four year old child, I was able to find an equation to finding the vital lung capacity of anyone using their height and age [14].  This equation is shown below as Equation 4.2.9.

Vlung [L] = 0.041(H [cm]) – 0.018(A [years]) – 2.69		(4.2.9)

Here, H is the height of the person in centimeters while A is the age in years.  Using Equation 4.2.9, I was able to find the vital lung capacities of multiple persons as shown in Table C.12B.  In obtaining the data shown here, I had to find some average statistics regarding heights of children and the average height of an adult man and woman.  For a child four years of age, the estimated height was 40 inches.  The average height of an adult man was 69.1 inches while that of a woman was 63.7 inches [15].  With all of these heights converted into centimeters, the average height of an adult was 168.7 centimeters.  Using these vital lung capacities, I was able to find an estimated ratio between the lung capacities of an adult to that of a four year old child.  This ratio was found to be about 2.4 to 1.  I then used this ratio for relating additional adult characteristics, to that of a four year old child.  More specifically, I used this ratio to calculate a child’s carbon dioxide exhalation rate per breath, and the volume of air exhaled per breath by a child.  These estimations are outlined in Table C.12C.

4.3 Conclusions

Repeating the approximations above for a child rather than an adult, Equations 4.2.1 to 4.2.9, I was able to find some interesting results when the SCC is implemented in a real-life scenario.  Using 100.0 cubic feet for the volume of an average vehicle sedan interior converted into 172800 cubic inches, a four year old child inside would need about 276 breaths after the guardian leaves to be detected by the SCC.  In terms of time in minutes, about 6.9 minutes would need to elapse after engine shut off before the child is to be detected by the SCC.  Considering that the inside of a vehicle reaches dangerous temperatures within 20 minutes, it seems that 6.9 minutes is very reasonable for such a real-life application.

5. COST

5.1 Parts And Labor

| Description | Part Number | Manufacturer | Qty. | Cost |
| --- | --- | --- | --- | --- |
| CO2 Sensor | TGS4161 | Figaro Engineering | 1 | $35.25 |
| Temperature Sensor | LM19CIZ | National Instruments | 1 | $0.18 |
| Operational Amplifier | TLC271 | Texas Instruments | 1 | $0.27 |
| Microcontroller | PIC16F877A | Microchip | 1 | $4.68 |
| Oscillator 20.0MHz | F1100E-0140 | Fox Electronics | 1 | $1.58 |
| Resistors: 10 Ω 120 Ω 470 Ω 1000 Ω 24000 Ω 330000 Ω | 25J10R 25J120R 4308R2471 25J1K 25J10K NA | Ohmite Ohmite Bourns Ohmite Ohmite Ohmite | 1 4 1 1 1 1 | $6.49 |
| Yellow LED 5mm 30 Degrees | HLMP3401 | Hewlett-Packard | 4 | $0.60 |
| SPST 5-POS DIP Switch | 76PSB05 | Grayhill | 1 | $1.26 |
| 12”x12”x6” Cardboard Box | NA | Walmart | 1 | $1.44 |
| 20”x12”x11” Cardboard Box | NA | Walmart | 1 | $1.59 |
| 12” Clear Plastic Wrap | NA | Reynolds Consumer Products | 1 | $2.19 |
| 5.6 V Zener Diode | D1N4734A | General Semiconductor | 3 | $0.30 |
| 0.1uF Capacitor | CK104 | BC Components | 5 | $1.27 |
| TOTAL |  |  |  | $64.81 |

| Labor | Rate | Hours | Cost |
| --- | --- | --- | --- |
| Robin O | $50/hour | 150 Hours | $7,500.00 |

Total costs for parts and labor = $7,564.81

6. CONCLUSIONS

Overall, I am very pleased with how the project turned out.  At first, I had a lot of uncertainties and doubts about this project.  I had never used carbon dioxide sensors before and I did not know at all how they detect carbon dioxide.  But after much researching, testing, and characterizing, I am much more knowledgeable and comfortable with using such sensors.

I am especially pleased to find the results mentioned in Sections 4.2 and 4.3.  Although for the purposes of the course, it is sufficient to succeed in the scaled-down project in itself, I am excited to know that this idea is something that may also be applied to a real-life scenario.  In addition to knowing that it may be applied to an actual vehicle interior with a four year old child, I know that if applied, it would not only work, but it would work well and time efficiently so as to prevent heat injuries and deaths.

Following the proper characterizations of the carbon dioxide sensor, and the other components, the project was tested, debugged, and eventually complete.  As I had set out to in the proposal and the design review, the SCC detects increases in carbon dioxide levels, detects changes in temperature, and performs the different actions to control the climate.

One thing to surely note is the versatility of the SCC.  It can be applied in many different aspects of everyday life.  Throughout this report, the SCC was project towards vehicles and saving the lives of young children.  However, it can also work for saving senior citizens, disabled people, and even valued animals or pets.  It can also be used for controlling the heater for use with cold weather.  Simple adjustments or additions to the software, and the SCC can accommodate for cold weather as well.  When the temperature falls below a certain level, the heating will be turned on for instance.

Also, the SCC idea may also be applied to homes or other buildings with multiple rooms and or floors.  As is usually the case, people do not tend to occupy every room in a building at every instance of time.  However, the ventilation system do not know this and blindly output heating or air-conditioning wasting large amounts of money on wasted energy.  However, with simple carbon dioxide and temperature sensors in place, rooms with no one present may be turned off from all ventilation while rooms with people present are automatically climate controlled.  This would reduce costs significantly in any application.

Lastly, the making of this project was very inexpensive in comparison to what was completed.  The most costly component was the carbon dioxide sensor.  However, in comparison to what the majority of carbon dioxide sensor costs were currently on the market, the cost of the Figaro TGS4161 was no burden at all.  Having a $35.00 part being the most expensive component of the whole circuitry, I am certain that in terms of real-life applications, the SCC would be the most reasonable.

REFERENCES

[1]	Hotel and Leisure Services Ltd., “A Car is Not a Child’s Toy: Preventing Heat Injury and Entrapment,” 2004, http://www.hotelfun4kids.com/travelnews/safetynews/car_safety-_heat.htm.

[2]	W. J. Smith, Radiometry and Photometry. New York: McGraw-Hill Inc., 1990.

[3]	Wikipedia: The Free Encyclopedia, “Carbon Dioxide,” June 2005, http://en.wikipedia.org/wiki/Carbon_dioxide.

[4]	Gastec Corporation, “Non-Dispersive Infrared Absorption Sensors,” 2001, http://www.gastec.co.jp/english/products/gas%20sensors/red_sensors.htm.

[5]	Telaire, CO2 OEM Module - 6004, Telaire, 2003.

[6]	Figaro Engineering Inc., TGS4161 – The Detection of Carbon Dioxide, Figaro, 2004.

[7]	National Semicondutor, LM19, National Semiconductor Corporation, 2003.

[8]	Microchip, PIC16F87XA Data Sheet, Microchip Technology Inc., 2003.

[9]	LED Supply, “Yellow LED 5mm 30deg,” 2004, http://www.ledsupply.com/l4-0-y5th30-1.html.

[10]	Arrick Robotics, “Driving High-Power Loads with a Microprocessor,” 2005, http://www.robotics.com/highload.html.

[11]	Family Practice Notebook, LLC., “Respiratory Rate,” 2000, http://www.fpnotebook.com/LUN54.htm.

[12]	Energy Services Publications, “Measuring Carbon Dioxide Inside Buildings,” 2001, http://www.wapa.gov/es/pubs/techbrf/co2.htm.

[13]	Pranayama – The Practice, “The Basic Breath,” June 2004, http://www.patanjali.ch/practice/breath/prana_1.html.

[14]	Balloons and Vital Lung Capacity, “Balloons and Vital Lung Capacity,” 2004, http://van.hep.uiuc.edu/van/qa/section/Everything_Else/Humans_and_Animals/977343689.htm.

[15]	About, Inc., “Average Height for a Woman Statistics,” 2005, http://pediatrics.about.com/cs/growthcharts2/f/avg_ht_female.htm.

# APPENDIX A – BLOCK DIAGRAMS/FLOW CHARTS

.

Diagram A.1 Block Diagram of the Smart Climate Controller

Diagram A.2 Flow Chart of the Embedded Software

# APPENDIX B – SCHEMATICS/MECHANICAL DRAWINGS

Figure B.1 Schematic of the Smart Climate Controller

Figure B.2A Figaro TGS4161 Carbon Dioxide Sensor

Figure B.2B Figaro TGS4161 Carbon Dioxide Sensor Cross-Section

Figure B.3A Driving Loads < 200 mA with a Microcontroller

Figure B.3B Driving Loads > 200 mA with a Microcontroller

Figure B.4A Model Type 1 Volume

Figure B.4B Model Type 2 Volume

# APPENDIX C – MANUFACTURER SPECIFICATIONS/TEST DATA

| Typical Detection Range | 350-5000 ppm |
| --- | --- |
| Heater Resistance | 70 Ω |
| Heater Current | 50 mA |
| EMF(350 ppmCO2) | 220 ~ 490 mV |
| ΔEMF(3500 ppmCO2) | 44~72 mV |
| Heater Voltage | 5.0 VDC |

Figure C.1 Figaro TGS4161 Carbon Dioxide Sensor Ratings

Figure C.2A Preliminary Simulation of Amplifier Circuit with 10V/V Gain

Figure C.2B Preliminary Simulation of Amplifier Circuit Results

|  | @ 350 ppmCO2 | @ 3500 ppmCO2 |
| --- | --- | --- |
| EMF (mV) | 170.625 | 112.700 |
| ∆EMF (mV) | 0.000 | 57.925 |
| Amplifier Output (mV) | 4090 | 2703 |

Figure C.3 Actual EMF Output Voltages from Carbon Dioxide Sensor

| Degrees (C) | Degrees (F) | Output Voltage (mV) |
| --- | --- | --- |
| 130 | 266 | 303 |
| 100 | 212 | 675 |
| 80 | 176 | 919 |
| 30 | 86 | 1515 |
| 26.4 | 79.5 | 1543.0 |
| 24.7 | 76.5 | 1562.5 |
| 25 | 77 | 1574 |
| 21 | 70 | 1604.9 |
| 16 | 60 | 1670.4 |
| 0 | 32 | 1863.9 |
| -30 | -22 | 2205 |
| -40 | -40 | 2318 |
| -55 | -67 | 2485 |

Figure C.4A LM19 Output Voltage Data

Figure C.4B LM19 Output Voltage Plot

| Trials | @ 4 seconds | @ 6 seconds | @ 8 seconds | @ 10 seconds |
| --- | --- | --- | --- | --- |
| 1 | 3724 mV | 3741 mV | 3580 mV | 3438 mV |
| 2 | 3774 mV | 3661 mV | 3580 mV | 3315 mV |
| 3 | 3870 mV | 3744 mV | 3462 mV | 3339 mV |
| 4 | 3641 mV | 3767 mV | 3487 mV | 3345 mV |
| 5 | 3825 mV | 3690 mV | 3584 mV | 3368 mV |
| 6 | 3741 mV | 3702 mV | 3610 mV | 3320 mV |
| 7 | 3804 mV | 3703 mV | 3517 mV | 3372 mV |
| 8 | 3781 mV | 3710 mV | 3480 mV | 3354 mV |
| 9 | 3768 mV | 3698 mV | 3521 mV | 3352 mV |
| 10 | 3713 mV | 3702 mV | 3601 mV | 3321 mV |
| Average | 3764 mV | 3712 mV | 3540 mV | 3352 mV |
| Estimated ppm CO2 | 426.8 | 640.2 | 853.3 | 1067.0 |

Figure C.5A Source Magnitude Test Using 12”x12”x6” Model (864 cubic inches)

Minimum Output Voltage = 2417 mV

| Trials | @ 4 seconds | @ 6 seconds | @ 8 seconds | @ 10 seconds |
| --- | --- | --- | --- | --- |
| 1 | - | - | 3919 mV | 3875 mV |
| 2 | - | - | 3975 mV | 3854 mV |
| 3 | - | - | 3979 mV | 3868 mV |
| 4 | - | - | 3960 mV | 3870 mV |
| 5 | - | - | 3954 mV | 3867 mV |
| 6 | - | - | 3976 mV | 3856 mV |
| 7 | - | - | 3970 mV | 3872 mV |
| 8 | - | - | 3965 mV | 3874 mV |
| 9 | - | - | 3980 mV | 3890 mV |
| 10 | - | - | 3969 mV | 3868 mV |
| Average | - | - | 3965 mV | 3869 mV |

Figure C.5B Source Magnitude Test Using 20”x12”x11” Model (2640 cubic inches)

# Figure C.6 Carbon Dioxide Sensor Output Voltages at 4-Second Exhalations

# Figure C.7 Carbon Dioxide Sensor Output Voltages at 6-Second Exhalations

# Figure C.8 Carbon Dioxide Sensor Output Voltages at 8-Second Exhalations

# Figure C.9 Carbon Dioxide Sensor Output Voltages at 10-Second Exhalations

# Figure C.10 Comparison of Average Voltages and Volumes

| Trials | 10 sec | 20 sec | 30 sec | 40 sec | 50 sec | 60 sec | 70 sec | 80 sec | 90 sec | 100 sec | 110 sec | 120 sec | 130 sec | 140 sec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4025 | 4025 | 3952 | 3909 | 3892 | 3888 | 3887 | 3887 | 3888 | 3889 | 3891 | 3894 | 3897 | 3901 |
| 2 | 3902 | 3725 | 3642 | 3606 | 3587 | 3579 | 3578 | 3580 | 3582 | 3584 | 3586 | 3589 | 3592 | 3596 |
| 3 | 4025 | 3850 | 3746 | 3699 | 3680 | 3675 | 3675 | 3678 | 3682 | 3685 | 3688 | 3691 | 3694 | 3698 |
| 4 | 4009 | 3802 | 3706 | 3657 | 3637 | 3632 | 3630 | 3632 | 3634 | 3638 | 3642 | 3646 | 3652 | 3659 |
| 5 | 3969 | 3801 | 3757 | 3742 | 3734 | 3727 | 3726 | 3728 | 3731 | 3736 | 3743 | 3752 | 3761 | 3770 |
| Average | 3986 | 3841 | 3761 | 3723 | 3706 | 3700 | 3699 | 3701 | 3703 | 3706 | 3710 | 3714 | 3719 | 3725 |

# Figure C.11A Response Time Test Using 12”x12”x6” Model

# Figure C.11B Response Time Test Using 12”x12”x6” Model

| Newborns | 44 breaths/minute |
| --- | --- |
| Infants | 20 – 40 breaths/minute |
| Preschool | 20 – 30 breaths/minute |
| Older Children | 16 – 25 breaths/minute |
| Adults | 14 – 18 breaths/minute |

# Figure C.12A Average breaths per minute for different age groups

| Age (years) | Estimated Vital Lung Capacity (liters) | Ratio to Child’s |
| --- | --- | --- |
| 4 | 1.4036 | 1:1 |
| 25 | 3.7750 | 2.6895:1 |
| 45 | 3.4150 | 2.4330:1 |
| 60 | 3.1450 | 2.2407:1 |

Figure C.12B Vital Lung Capacities

|  | Average Adult | Average Child |
| --- | --- | --- |
| Vital Lung Capacity | 3.4150 L | 1.4036 L |
| Breaths Per Minute | 16 | 30 |
| CO2 Exhalation | 42500 ppm CO2/breath | 17468 ppm CO2/breath |
| Exhalation Air Volume | 500.0 mL/breath | 205.5 mL/breath |

Figure C.12C Estimated and Scaled Measurements

# APPENDIX D – PICTURES

Figure D.1 Actual Circuit Board Containing Carbon Dioxide Sensor

Figure D.2 Actual Circuit Board Containing Temperature Sensor and Execution Switch

Figure D.3 The Complete Smart Climate Controller