Smart Climate Controller

ECE 445 Senior Design

Project Proposal

June 16, 2005

Project Team

Robin O

Teaching Assistant:

Timothy O’Connell

INTRODUCTION

Background

In these technologically advanced times where cars can now come with navigational systems and rear view cameras, it is quite disturbing to know that nothing much is being done to prevent heat injuries to children locked in cars.  More specifically, during the warmer summer days, numerous children are injured and dying from being left unattended inside a parked car.  Every year, according to the National Safe Kids Campaign, about 30 children die from heat-related illnesses after being left unattended in parked cars.  A small child’s body temperature can increase three to five times as fast as an adult’s.  When the outside temperature is 93°F, even with a cracked window, the temperature inside a car can reach 125°F in just about 20 minutes and 140°F in 40 minutes.  Of course, the obvious thing to do is to not leave children unattended in cars.  However, even after all this time and all the warnings, these disasters occur at an outstanding rate.  Whether it is from a caretaker’s ignorance, or from a child crawling into an unlocked car sitting on the driveway, the risk is out there and if possible something should be done to nullify all of these possible risks.

Objectives

The objective of the Smart Climate Controller, or SCC, is to provide automated ventilation and thus to eliminate such tragedies.  Assuming the car is initially running, the SCC is activated one minute after the owner’s key has been removed from the ignition keyhole.  At this point, a carbon dioxide sensor located inside the car monitors the carbon dioxide (CO2) concentration level and watches for any increase.  Since if no one is inside the car, the CO2 level can only decrease, an increase signifies that someone has been left behind and the SCC triggers a thermostat.  A temperature sensor also located inside the car looks for any dangerous increase in temperature by comparing values to a preset limit.  When the temperature meets this preset limit, the ignition is started, the air-conditioning is turned on, the windows are rolled up if not already, and the doors are locked if not already to prevent theft.  This system is disengaged when an actual key has been inserted into the ignition keyhole signifying that the car owner has returned.  Rather than setting off an alarm that only serves to annoy and deter people away from the car, the SCC serves to temporarily reduce dangerous risks to a child inside a car.  Also the SCC serves to protect not only children but elders and also valued pets.  Furthermore, the SCC may be applied to a home or a corporate building environment to save energy and wasted ventilation services to unused rooms.  Such CO2 sensors may be used to monitor the number of persons in different rooms and adjust the heating and or air-conditioning distribution correspondingly.  It can shut it off completely if no one is in a particular room at all.  The following page lists some of the SCC features and benefits.

Product Features

  - Uses internal thermostat and air-conditioning to regulate set temperature

  - Powers directly from the car’s 12-V battery

  - Easily cleanable and accessible carbon dioxide and temperature sensors

  - Reads 0 - 2000ppm CO2 in 32 - 122°F temperatures

  - Automatically self-engages at engine shut off and disengages with key in ignition

Product Benefits

  - Protects children, elders, and valued pets

  - Offers peace of mind with a major reduction of risks of heat injuries to loved ones

  - Maintains the security of the car by locking doors and closing windows

  - Fully automated and easily disengaged

  - Built into car so there are no messy wires or use of the power adaptor/cigarette lighter

DESIGN

Block Diagram

Figure 1: Block Diagram

Block Diagram Description

Execution:  The Execution block models the portion of the circuit which engages or disengages the overall circuit of the SCC.  The SCC engages one minute after the time when the Execution block recognizes that the owner key has been removed from the ignition keyhole while it disengages once the key has been re-entered.  The Execution block’s information is sent to the Control Unit of the Microcontroller block for further processing.  This block is powered by the 12-V car battery not shown in the figure shown.

Sensor Circuits:  The Sensor Circuits block consists of the circuits necessary to accommodate both the Temperature and the Carbon Dioxide Sensors which are both placed inside the car.  Once engaged, the Carbon Dioxide Sensor will continuously monitor the CO2 levels inside the car and send the information to the Control Unit of the Microcontroller block.  The Control Unit will then monitor for any increase in CO2 levels.  The Temperature Sensor will also continuously track the inside temperature of the car and send the corresponding information to the Thermostat of the Microcontroller block.  The Sensor Circuits block will be powered by the 12-V car battery not shown in Figure 1.

Microcontroller:  This block is the heart of the SCC and consists of the Thermostat and the Control Unit.  Once the Control Unit receives information to engage from the Execution block, readings from the Carbon Dioxide Sensor are first monitored for any significant increase in CO2 levels.  Such an increase signifies a child is present inside the car and triggers further action.  At this point, the Control Unit awaits communication from the Thermostat.  The Thermostat observes the temperature via the Temperature Sensor of the Sensor Circuits block.  It continuously compares the temperature reading to a preset upper limit of 70°F.  Once this upper limit has been exceeded, it notifies the Control Unit.  The Control Unit then signals the Operations block to turn the ignition on.  Soon after, the Control Unit instructs for the air-conditioning to turn on while the doors are locked and the power windows are rolled up.  In the meantime, the Thermostat continues to examine the temperature readings in comparison to the preset lower limit value, ten degrees in Fahrenheit lower than the upper limit.  Once the temperature falls to the preset lower limit value, in this case 60°F, instructions to turn the air-conditioning off are sent to the Control Unit and the Operations block to save energy.  However, once the temperature reaches the preset upper limit again, the air-conditioning is turned on once again as the cycle continues until the Execution block signals the owner’s return.  The Microcontroller block is also powered via the 12-V car battery and necessary circuitry.  The microcontroller to be used is a PIC16F877A because it offers 33 input/output ports which will accommodate for the above Operations and any additional Operations that may be added at a later time.  Its eight 10-bit analog to digital converters accommodates the two sensors used here while also allowing additional sensors for future uses.  In addition to these factors, its low cost and ease of use make it a great candidate for the SCC.

Operations:  The Operations block simply encases the actions to be taken when someone is determined to be inside the car while the engine is off.  It entails turning on the ignition to allow for the air-conditioning.  The doors are locked and the windows are also rolled up to prevent deter theft while the engine is running.  In Figure 1, the air-conditioning block is isolated because once the Operations block is engaged, it is constantly turned on and off until the owner returns or until the inside temperature of the car saturates well below the lower preset limit.  These elements are also run via power from the 12-V car battery.

Performance Requirements

  - Power consumption is small enough such that the car battery is not significantly affected

  - Sensor readings are taken promptly to take actions before any possible heat related injuries (one minute after owner key is taken out of the ignition keyhole)

  - Sensor readings are taken at least in one-minute intervals

  - Power windows are rolled up in one-second intervals to prevent injuries while closing

  - Detects the presence of a person inside the car no less than 99% of the time

  - The air-conditioning with full capacity, and the ignition turns on 99% of the time

  - The doors do not lock when the key is sensed in the ignition keyhole

  - The product is built into the car when manufactured to avoid manual removal and manual disengagement

VERIFICATION

Testing Procedures

The Carbon Dioxide Sensor will initially be tested in a variety of different settings.  It will be tested mainly for/against temperature, the size of the surrounding environment, distance of the source, size of the source, speed, consistency over time, and reliability.  For these tests, a known sample of CO2 concentration will be used consistently.  Possible sources consist of carbonated soda bottles, large insects, a consistent human subject, or the like.  In each case, the size of the surrounding environment will be kept constant and or variable controlled.  The Temperature Sensor will also be tested for functionality; however the process will be much simpler than that of the Carbon Dioxide Sensor.

Temperature:  The output of the Carbon Dioxide Sensor will be tested via a voltmeter at varying temperatures.  In a consistent environment, the output will be recorded at approximately 50°F, 70°F, and 90°F.  In each case, five trials of data will be obtained and recorded.  Because during the actual implementation, the Carbon Dioxide Sensor ideally needs only to sense at approximately 70°F, this range should be adequate.  The Temperature sensor will be used during these tests to monitor these temperatures.  Heat will be applied via a light bulb considering that the tests will be conducted inside a small environment.  More specifically, I will obtain a graph showing relationships of CO2 concentration versus temperature in a given volume with a consistent CO2 source.  Deviations found here will be used to construct a graph relating CO2 concentration error versus temperature.

Size of the Surrounding Environment:  Because the size of an environment is such an important factor with concentration measurement, it must be tested for sensitivity.  For the scope of the course, I plan to use a small enclosed box with a few ventilation holes for my tests.  The approximate dimensions of the box are 1’ x 1’ x 1’.  However, the Carbon Dioxide Sensor will also be tested in larger areas such as an apartment bathroom, an apartment bedroom, and lastly the senior design laboratory.  Five trials of data in each environment will be recorded and observed for consistencies.  Graphs comparing CO2 concentrations versus environment volume given a consistent source will be constructed.  From this a relationship can be formed and used for a more approximate emulation of a car’s interior.

Distance from Source to Sensor:  In a consistent environment, the Carbon Dioxide Sensor’s decay in sensitivity will be observed as a function of increasing distance.  The maximum allowable distance, to be determined at a later time, will be divided into five equal intervals of length and measured five times at each interval.  A graph will be formed from the data in terms of the error of the sensor output as a function of distance.  Thus relationships of deviations in CO2 concentration versus distance in a given volume with a consistent source will be observed.  This will help to determine the ideal location of the sensor with respect to a passenger.

Sensor Reaction Time/Speed:  Each of the two sensors will be tested for optimum reaction times.  Using the microcontroller, output from the sensors will be sampled at different intervals of time.  For instance, readings will be accessed every 20 seconds, 40 seconds, and every 60 seconds.  The results will be compared to see if there is any affect on accuracy as a factor of time allotted for readings.  Graphs relating the deviation from the CO2 concentration measured at 60 seconds versus the time allotted for each read will be constructed.  Graphs relating the deviation from the temperature measured at 60 seconds versus the time allotted will also be constructed.  If there is no significant affect, the shortest interval of 20 seconds will be used.  Since CO2 by nature will most likely not accumulate or dissipate in significant amounts in an enclosed area so quickly, intervals shorter than 20 seconds seem unnecessary.

Consistency over Time:  The Carbon Dioxide and the Temperature Sensors will be tested for consistency over time.  They will be run continuously to observe any drift in performance.  For ten minutes, the CO2 sensor will be tested at one-minute intervals with a new and consistent source of CO2.  The Temperature Sensor will be compared to a thermometer for any inconsistencies with time.  Data from this will form graphs relating output error as a function of continuous runtime.  More specifically, the graphs will relate deviations from initial CO2 concentrations and temperature versus time in a given volume.  These relationships will show the optimum times to run future tests and eventually the final product.

Tolerance Analysis

As suggested earlier in the section, the most important component of the product is the Carbon Dioxide Sensor.  It is the biggest factor regarding proper functionality of the SCC system.  The sensor must be sensitive enough to detect the slightest increase in levels of CO2 concentration in both a smaller emulated environment and also in an automobile interior.  This is especially the case with babies where the release of CO2 is relatively low.  Rough ratios of environment size to the size of the CO2 releasing agent will be taken into consideration and tested thoroughly.  It is believed that following the tests mentioned in the Testing Procedures section will result in a much better notion of how to achieve the optimum performance.

More specifically, to measure the Carbon Dioxide Sensor’s sensitivity to the increase in levels of CO2 concentration, the following test will be run.  Five tests will be run using five different enclosed testing environments varying in volume.  For each volume, the Carbon Dioxide Sensor will be run continuously while connected to a voltmeter to its terminals.  Using a straw or the like, air, and thus CO2, will be exhaled into the enclosed area for five seconds.  To promote consistency, ten such trials will be made for each volume with sufficient venting between each trial.  After allowing 60 seconds for each reading, the voltage displayed via the voltmeter will be recorded.  This procedure will be repeated for each of the other four volumes.  Using such data, it is possible to detect to what level of sensitivity the Carbon Dioxide Sensor can sense increases in concentration levels.  Because this project focuses on detecting increases in CO2 levels and not on the actual concentration values, these tests are thought to be sufficient ratings for the sensor.

COST & SCHEDULE

Cost Estimates

| Labor |  |
| --- | --- |
| Robin O ($50/hr) x 2.5 x 100 hrs | $12,500 |
| Parts |  |
| Analog Circuit Components - Estimate (resistors, capacitors, inductors, op amps, wires, breadboard, etc) | $10.00 |
| Simulation Box (Five 1/8” 12”x 12” clear acrylic sheets) | $9.79 |
| Carbon Dioxide Sensor Module (Telaire CO2 OEM Module 6004) | $149.00 |
| PIC Microcontroller 16F877A | $4.68 |
| Semiconductor Temperature Sensor (Dallas Semiconductor IC DS620) | $1.76 |
| Other Accessories – Estimate (sealant, tape, glue, screws, etc) | $15.00 |
| Solder Accessories | $20.99 |
| 6” x 9” Inch Breadboard | $35.90 |
| TOTAL | $12,747.12 |

Schedule

| Week | Description of Activities |
| --- | --- |
| 6/20 | - Order CO2 Sensor – Modules from Telaire or Figaro - Get temperature sensor from ECE Parts Shop – Semiconductor Sensor - Begin assembly of simulation box – From Illini Plastics or Machine Shop 5 sheets of 1/8” 12” x 12” clear acrylic - Design Sensor Circuits – Meeting specifications of datasheets - Begin coding of PIC16F877A – Recognizing sensor inputs via ADC & outputting to Operations block |
| 6/27 | - Test CO2 and temperature sensors thoroughly following Testing Procedures – Find optimum testing parameters: time, distance from source, environment size - Begin actual build – Attach sensors to box and finish circuitry - Finish coding of PIC16F877A – Test for functionality |
| 7/4 | - Debugging – Test for accuracy and proper sequencing of events/operations |
| 7/11 | - Debugging and final tests – Test for Performance Requirements |

Overheating in Cars: Never Leave Your Child Alone, National Safe Kids Campaign