Explanatory note

The aim of our calculations is to determine the overpressure resulting of an explosion of cereal duts in a vessel. For this, we have apply the following method :

- Calculation of the burning dusts mass

Considering the volume of the vessel and the concentration of dust, we calculate the masse of dusts which can burn during the explosion.

Thanks to the ligne “% of dusts burned”, it’s possible to considere that not all of the dusts burn.

In our example, we take into consideration a vessel of 1200 m3, a dust concentration of 65 g/m3 (lower explosive limit) and the fact that all the dusts burn.

- Calculation of the quantity of gaz created

We consider the following burning equation :

C50H6O44 + 29,5 O2   50 CO2 + 3 H2O

The molar mass of dust is 1310 g/mol (= 50 x 12 + 6 x 1 + 16 x 44). Consequently, thanks to the burning dusts mass, we deduce the quantity in mole of dust and O2 burned and the quantity in mole of CO2 and H2O created.

Therefore, we know the quantity in mole of gaz created (mole of CO2 created + mole of H2O created – mole of O2 burned) and we calculed the volume of gaz created thanks to the molar volume (24 L/mal at 20°C).

The gaz created are about 1000°C. So, we calculate the volume of the gaz at 1000°C (= volume à 20°C x (273 + 1000) / (273 + 20)).

- Calculation of the overpressure in the vessel without vent

To calculate the overpressure in the vessel, we consider that the overpressure’s increase is proportional to the increase of the gaz volume.

For example, in our case, the volume of gaz created at 1000°C is 146 m3. It represent 12,1% of the vessel’s volume. Therefore, we consider that the overpressure is 12,1 % of the initial pressure, 1 bar in our example.